ASIOHost DLL Overview

ASIOHost provides a full duplex interface between your professional audio application and the sound card.  ASIOHost supports the latest ASIO 2.2 standard, and supports as many channels as the system can manage at full sample bit width. It  bridges the device driver's real time buffer servicing interrupt demands to Window's unpredictable file I/O. Buffer servicing for the ASIO driver is isolated from buffer servicing in the application such that in no way can an inordinate delay in application side buffer servicing cause a hold up in ASIO's real-time buffer switching thread, or crash the ASIO driver. ASIOHost's innovative design, together with *common sense platform optimization yields a high level of confidence in capturing clean, sample accurate professional audio without stressing the device.

(*While beyond the scope of this discussion, common sense platform optimization begins with simple measures such as disabling  anti-virus software and Windows search indexing during recording sessions. More information on this topic can be found on sound card manufacturer's web sites.)

Any sample rate supported by the sound card may be used, and there are functions supplied to get and set the sample rate. The default sample bit width expects 16 bit audio data from the application and returns 16 bit samples to it. Beyond this, you may choose to employ the device's sample bit width on a per channel basis, and you may have different bit widths for play and for record on any given channel. For example, on a sound card that runs natively at 32 bits per sample, you may chose some channels to employ this bit width while others run at 16 bits per sample, or perhaps play 16 bit samples on one track while recording a track in sync to that at the full 32 bits per sample resolution.

ASIOHost supports sound cards with 16, 24, or 32 bit samples, converting the sample bit width between the application and the ASIO sound buffers on the fly as required. Volume control is accomplished without cost via simple bit shifting done at the same time as sample bit rate conversion for high performance in the real-time priority ASIO buffer servicing thread. Bit shifting audio results in a 6 dB change in volume per increment/decrement, and 16 levels of audio are provided. However, user may elect to optionally provide finer control of audio volume before it is sent to ASIOHost instead. (Available upon request.)

Operational Modes

ASIOHost features various modes that can be selected to present a sound card's inputs and outputs to the application individually, or in stereo pairs. Thus the sound card is mapped to "virtual channels" according to the mode. A set of virtual channels can be record only, or play only, or full duplex for playing and recording at the same time. The application must choose the desired mode at app init time.

The modes available are:

MONO_PLAY_ONLY,		MONO_RECORD_ONLY,	MONO_PLAY_RECORD,

STEREO_PLAY_ONLY,	STEREO_RECORD_ONLY,	STEREO_PLAY_RECORD.

It is expected that the application will only be interested in a particular mode, such as  MONO_PLAY_ONLY for a PA (Public Address) system, or  STEREO_PLAY_ONLY for a music player, while a sound editor may want STEREO_PLAY_RECORD.

Application Interface Overview

There are only a few steps required to be up and running with ASIO audio. These steps are:

- Select the desired ASIO device and initialize the ASIO system.

- Select the mode and create the ASIO buffers.

- Create a virtual channel, passing creation parameters and buffer servicing callbacks from your application.

- Signal the DLL to prepare and pre-load buffers with audio data via your callback functions.

- Call Start for each virtual channel.

- While running, you can call Pause and Resume, SetVolume and GetPosition.

- Call Stop when you are done.

- Close virtual channels.

- Call shut down on the ASIO system at application exit time.

A variety of functions are provide to configure options and retrieve information, such as...

- Get ASIO devices available and select desired device.

- Get/Set sample rate.

- Get the primary ASIO buffer size.

- Get device sample bit width, Set application sample bit width per track.

- Get max number of virtual channels the device will provide for chosen mode.

- Get input/output latency samples.

- Get device channel names for display.

- Get/Set max virtual buffers.

- Get/Set output volume level on a per channel bases.

Exported Functions

To provide flexibility to create  very wide range of applications, a total of 24 functions are exported from the current version of the DLL as follows...

extern int ASIOGetDevices(char *pStrBuf, int bufsize);

extern bool ASIOInit(HWND hwndMain, int nDevNum);

extern void ASIOShutdown(void);

extern bool ASIOSetSampleRate(DWORD dwSampleRate);

extern DWORD ASIOGetSampleRate(void);

extern int ASIOGetPrimaryBufferSizeSamples(void);

extern BITDEPTH ASIOGetBitdepth(void);

extern int ASIOGetMaxVirtualChannels(MODE proposedMode);

extern int ASIOCreateBuffers(int nMaxVirtualChannels, MODE requestedMode);

extern DWORD ASIOGetOutputLatency(void);

extern DWORD ASIOGetInputLatency(void);

extern bool ASIOGetChannelName(char *name, int channel);

extern void SetMaxVirtualBuffers(int nMaxVirtualBuffers);

extern int GetMaxVirtualBuffers(void);

extern bool CreateVirtualChannel(LPCHANNELVARS pChannelVars);

extern void CloseVirtualChannel(int nVirtualChnNum);

extern bool PreLoadBuffers(int nVirtualChnNum);

extern bool StartPlay(int nVirtualChnNum);

extern void StopPlay(int nVirtualChnNum);

extern void SetVolume(int nVirtualChnNum, int level);

extern int GetVolume(int nVirtualChnNum);

extern DWORD GetPosition(int nVirtualChnNum);

extern void Pause(int nVirtualChnNum);

extern void Resume(int nVirtualChnNum);

Interfacing with ASIOHost

The first step is to initialize the ASIO system via ASIOInit with the application's main window handle and the desired device. However, before you can call that, your app needs to decide upon which ASIO device to open. Most likely, your application will let the user choose a device. To do this, you will need to create a dialog with a drop down list of devices (it must not sort). You can populate that list with a call to...

int  ASIOGetDevices(char *pStrBuf, int bufsize); //  bufsize: Size of buffer passed

This function returns the number of devices, and for each device it places that device's name in the char buffer passed, separating each with a NULL (0). You pass a char buffer big enough to hold a number of strings identifying devices, and the size of that buffer in parameter 'bufsize', as in this example from the SelectDevice dialog box procedure from ASIOHostDemo...

// *Note - devices combo must not sort!

LRESULT CALLBACK SelectDevice(HWND hDlg, UINT message, WPARAM wParam, LPARAM lParam)

{

HWND hWndCombo;

char *pStrBuf, *ptr;

int n, nDevices;

switch (message)

{

case WM_INITDIALOG:

pStrBuf = new char [2048];

if(pStrBuf == NULL)return TRUE;

ZeroMemory(pStrBuf, 2048);

nDevices = ASIOGetDevices(pStrBuf, 2048);

if(!nDevices)

{

delete [] pStrBuf;

return TRUE;

}

hWndCombo = GetDlgItem(hDlg, IDC_COMBO1);

ptr = pStrBuf;

for(n = 0; n < nDevices; n++)

{

SendMessage(hWndCombo, CB_ADDSTRING, 0, (LPARAM) (LPCTSTR)ptr);

// Device strings are separated by a NULL. Skip to next...

ptr += strlen(ptr);

++ptr;

}

delete [] pStrBuf;

return TRUE;

case WM_COMMAND:

if(LOWORD(wParam) == IDOK)

{

hWndCombo = GetDlgItem(hDlg, IDC_COMBO1);

nChosenDevice = SendMessage(hWndCombo, CB_GETCURSEL, 0, 0);

if(nChosenDevice >= 0)

{

// Now we have the desired device, go ahead and initialize ASIO...

bASIOInitialized = ASIOInit(hwndApp, nChosenDevice);

}

EndDialog(hDlg, LOWORD(wParam));

return TRUE;

}

if(LOWORD(wParam) == IDCANCEL)

{

EndDialog(hDlg, LOWORD(wParam));

return TRUE;

}

break;

}

return FALSE;

}

You will note above that once we had the chosen device number to initialize the ASIO driver for that device we called...	bool ASIOInit(hwndApp, nChosenDevice);

This is the first step required for initialization.

Other Options to Set the Device

Your application may simply pass a hard coded device number via the nDevNum parameter in early stages of development, or it may use a configuration file called “ASIOHost.txt” with the desired device number as the first character in that file. The file must be in the folder with ASIOHost. ie: it would have an ASCII zero ('0') for device 0, or an ASCII 1 ('1') for device 1, etc., as the first character.

Another option is to "install" the  The Multistream ASIO Player from

http://www.tropicalcoder.com/MStreamPlayer.htm

and select the device in here. This application then makes a registry entry that will be read by ASIOHost DLL. If your application wants to make use of the Windows Registry, the key it currently looks for is...

"HKEY_CURRENT_USER\\Software\\Custom Taylored Software\\MStream"

Options\\Devnum"; ProfileInt (REG_DWORD, REG_BINARY)

This may be modified upon request.

If you pass -1 in parameter nDevNum, ASIOHost will look for the device number first in the registry, and if it doesn't find it there it will look for the “ASIOHost.txt” configuration file, and if it doesn't find that it will try to open device 0.

Now once you have initialized ASIO for the desired device, there are a number of things you can do.

You may get and set the sample rate with:

extern bool ASIOSetSampleRate(DWORD dwSampleRate);

extern DWORD ASIOGetSampleRate(void);

You may read and display latency values with:

extern DWORD ASIOGetOutputLatency(void);

extern DWORD ASIOGetInputLatency(void);

You may retrieve other information (though these will be called later in the demo):

extern int ASIOGetPrimaryBufferSizeSamples(void);

extern BITDEPTH ASIOGetBitdepth(void);

Or you may decide how many virtual channels you are going to want for a given mode. You will ensure that the number of channels you propose to use is not more than the device can provide with a call to...

extern int ASIOGetMaxChannels(MODE proposedMode);

Then the next required step in the initialization sequence is to set the mode as the context under which the ASIO buffers will be created for the device. Your application most likely only uses a certain mode, rather than allow the user to choose a mode, as done in the demo app. The mode is set and the buffers are created with a single function call...

extern int ASIOCreateBuffers(int nMaxVirtualChannels, MODE requestedMode);

Once you created the ASIO buffers you may call...

extern bool __cdecl ASIOGetChannelName(char *name, int channel);

Though from now on we are dealing with “virtual channels”, for which there is a one-to-one correspondence with the actual output/inputs if we are in a mono mode, but are addressed as pairs in stereo modes, you may want to get the name of each each discrete channel you created if want to display them. You could get the channel names like this...

int n = (bMonoMode ) ?  nChannelsRequested : (nChannelsRequested * 2);

char **pChannelNames = new char* [n];

for(int i = 0; i < n; i++)

{

pChannelNames[i] = new char [32];

pASIOAudio->ASIOGetChannelName(pChannelNames[i], i);

}

Playing and Recording

Now we are ready to actually play or record some audio.

Look at the code in ASIOHostDemo.cpp. The steps required to begin playing and recording  are as follows...

First, we create functions that will fill the sound buffer with data during each callback. We will pass the address of these functions when we create the virtual channels. In ASIOHostDemo.cpp we have functions...

void RefillLeftBuffer(LPBYTE pSndBuf, DWORD dwBufferSizeBytes, DWORD dwInstanceData);

void RefillRightBuffer( LPBYTE pSndBuf, DWORD dwBufferSizeBytes, DWORD dwInstanceData);

void RefillStereoBuffer( LPBYTE pSndBuf, DWORD dwBufferSizeBytes, DWORD dwInstanceData);

We will also need a function that will receive captured data, interleave it if stereo and write it to file. In ASIOHostDemo.cpp we have functions...

void SaveLeftBuffer( LPBYTE pSndBuf, DWORD dwBufferSizeBytes, DWORD dwInstanceData);

void SaveRightBuffer( LPBYTE pSndBuf, DWORD dwBufferSizeBytes), DWORD dwInstanceData;

void SaveStereoBuffer( LPBYTE pSndBuf, DWORD dwBufferSizeBytes, DWORD dwInstanceData);

Now we need to call CreateVirtualChannel for each channel, passing the above buffer servicing functions to it so they may be called  at callback time. Its function prototype is...

extern bool CreateVirtualChannel(LPCHANNELVARS pChannelVars);

It takes a pointer to a ChannelVars struct defined in ASIOHost.h This structure contains fields for each parameter needed for the full specification of a channel, ie:

typedef struct channelvars_tag

{

int nVirtualChnNum;                                     // <--0 through  (nMaxVirtualChannelsAvailable - 1)

int nBufSizeSamples;                                    // <--Big enough for at least 1/10 second of audio samples

LPPROCESSBUFFER pOutBufServiceProc;     // <--Output buffer servicing callback

LPPROCESSBUFFER pInBufServiceProc;       // <--Input buffer servicing callback

DWORD dwInstanceData;                              // <--Application data, ie: object reference, chn num

bool bUseDeviceBitWidthPlay;                    // <--Must be false for free version of ASIOHost

bool bUseDeviceBitWidthRecord;                // <--Must be false for free version of ASIOHost

short reserved;                                               // <--Set to 0

}ChannelVars, *LPCHANNELVARS;

bool bUseDeviceBitWidthPlay and bool bUseDeviceBitWidthRecord are how you specify that the channel should use the sound card's bits per sample for play and/or record. If you set these to false, your application will be using 16 bits per sample. To know what the sound card's bit depth is, you may call extern BITDEPTH ASIOGetBitdepth(void); It returns a type “BITDEPTH” as defined in the enum by that name at the top of ASIOHost.h

Whatever you passed in the dwInstanceData parameter will be returned you with every callback, whether for output data or captured data. For example, to pass an object reference for later use at callback time do:  dwInstanceData = (DWORD)(LPVOID)this.

An LPPROCESSBUFFER is a pointer to a void function that takes an LPBYTE and two DWORD parameters, as defined at the top of ASIOHost.h

typedef void (*LPPROCESSBUFFER) (LPBYTE, DWORD, DWORD);

You will need unique buffer servicing functions for each virtual channel you create. You can see that the above functions meet this definition. You can see how CreateVirtualChannel  is employed in ASIOHostDemo.cpp We call it in local function void PrepareVirtualChannels(void); If only playing or only recording, you can pass NULL for the unused  LPPROCESSBUFFER field. It will be ignored in any event. The first dword returns buffer size bytes, while the second returns whatever dwInstanceData you passed at channel creation time.

Let's talk a bit about the field  int nBufSizeSamples in struct ChannelVars...

In nBufSizeSamples, you will pass the chosen sound buffer size. ASIOHost will create a number of buffers (the default number of buffers created is 16) of this size.  This is the size of the buffer that you will be filling with audio samples during each callback if playing, or reading samples from if recording.

The idea is that while you are filling or reading one of these buffers, the ASIO device has several others to use while you are servicing the current buffer. A good size would be about 100 milliseconds of audio samples. 16 of these, the default, would mean that you are buffering 1.6 seconds of audio ahead, which should be sufficient to handle even the worst delays that can be thrown at you by the system during buffer servicing. You may increase or decrease the number of buffers created by calling SetMaxVirtualBuffers() after you did ASIOCreateBuffers but before any calls to CreateVirtualChannel, and you can determine how many were created by calling GetMaxVirtualBuffers(). Be advised that during the call to  PreLoadBuffers(), you output buffer serving function will be called this many times to fill each buffer with an initial load of data, for each channel.

The size and number of buffers chosen has no impact on latency, which is a factor determined by the ASIO buffer size you will have chosen in the ASIO Panel for your sound card. A typical value for this is 256 samples, though many sound cards permit you to go down as low as 64 samples, or even 32. This value is chosen in the sound card's panel such that it is high enough that you don't hear any crackling sound or experience dropped buffers. In the source code and the discussion below this is referred to as “the primary buffer size”.

Now I said that your sound buffer size should be about 100 milliseconds of audio data, but it must also be some exact multiple of the primary buffer size (the size selected in the ASIO Panel for the device).

Assuming a sample rate of 44100 samples per second, 100 milliseconds at that rate would be 4410 samples, so your sound buffer should be around that value. Now what if you don't know what primary buffer size may be used? Then you may compute sound buffer size at run time this way...

// Calculate how many samples in 100 milliseconds at current sample rate...

DWORD dwBufferSampleSize = ASIOGetSampleRate() / 10;

// Now ensure we have an exact multiple of the current primary buffer size...

DWORD dwPrimaryBufferSampleSize = ASIOGetPrimaryBufferSizeSamples();

DWORD n =  dwBufferSampleSize /  dwPrimaryBufferSampleSize;

if((dwBufferSampleSize % dwPrimaryBufferSampleSize) != 0)++n;

if(n < 2)n = 2;  // Must be at least twice the size of the primary buffer

dwBufferSampleSize = n * dwPrimaryBufferSampleSize;

If you are feeding input through to output, then you will want as small a buffer as possible...

dwBufferSampleSize = 2 * dwPrimaryBufferSampleSize, and will set  SetMaxVirtualBuffers to a small value.

Now there is something about choosing and working with a given sound buffer size that has not yet been mentioned. ASIOHost expects monaural audio data, so for stereo the left and right channel data must be de-interleaved and passed side by side in a buffer, left channel data followed by right channel data. Stereo input data will be returned in a similar manner. That is, when in a stereo mode, each buffer will actually be 2 buffers of this size, side by side. Another way of stating that is: Stereo buffers will be twice the chosen size. However, all through the code only the size of a mono buffer will be given or passed or referred to. The rule is that if you are in a stereo mode, you may assume that the right channel samples follow dwBufferSampleSize left channel samples within the buffer passed.

Looking at the code in ASIOHostDemo.cpp...

void RefillStereoBuffer(LPBYTE pSndBuf, DWORD dwBufsizeBytes, DWORD dwInstanceData)

{

GetStereoSamples(dwBufsizeBytes / PlayBytesPerSample, pSndBuf, (pSndBuf + dwBufsizeBytes));

}

...you can see that we are passed buffer size bytes during the callback. We calculate num samples via  dwBufsizeBytes / PlayBytesPerSample, and create pointers to left and right channels with  pSndBuf  for the left channel, then use dwBufferSizeBytes to index over to the right channel portion of  pSndBuf (at pSndBuf + dwBufferSizeBytes) and we will copy buffer size bytes of right channel data to that. The lesson to take away from here is that parameter dwBufferSizeBytes is only the size of the left or right half of the sound buffer when we are in stereo mode.

After creating the virtual channels, we need to prepare and pre-load the buffers with audio data, so that when we call StartPlay for each channel they will all start at the same time without each being delayed during the time it takes to fill all the buffers, before the next channel can be started. We do this by calling...

bool PreLoadBuffers(int nVirtualChnNum);

...for each channel. Then we call StartPlay() for each channel.

Once we called StartPlay, we may call Pause whenever we wish to stop and Resume when we want to continue. When we are done playing or recording we will call StopPlay(). If we want to do more we only need to call StartPlay() once more, but don't forget to call PreLoadBuffers() for each channel  again first.

If getting and display position and you are running two or more virtual channels in sync, only call for and display the position for one of those channel and let that stand for all, because a call for each will return a different position for each. This is because positions returned approach sample accuracy, and therefore only reflect the position at the exact moment called, and it is impossible to call for two or more positions at exactly the same time.

On app shutdown, we must call CloseVirtualChannel() for each channel we created earlier with calls to CreateVirtualChannel, and besides closing any open files and de-allocating any local buffers we will do...

ASIOShutdown(); ...to release the ASIO driver.

What is this WM_WANTS_RESET?

In ASIOHost.h the first thing defined is a private Windows message...

#define WM_WANTS_RESET (WM_APP + 10)

The ASIO specification permits the driver to request a reset anytime it feels the need, but I have never seen this happen. Also, when your application asks for a change in sample rate, some few sound cards require a complete reset in order to comply. In these cases ASIOHost will send the WM_WANTS_RESET to your application's Windows message loop. You must respond to such a message by posting a message box asking the user to close and restart the application. When the application is restarted, if you requested a sample rate change, the sound card's sample rate will be the requested rate if it is one of the sample rates supported by that device.  The following is an example of code you will find in ASIOHostDemo.cpp...

case WM_WANTS_RESET:

// A sound card may request a reset if it requires one.

// Some sound cards require a complete reset when an application requests a change in sample rate

// In this case, ASIOHost will send a WM_WANTS_RESET message, as defined in ASIOHost.h

MessageBox(NULL, TEXT("Please exit the application and restart it for  change to take effect.\0"), TEXT("Sample rate change request...\0"), MB_OK |  MB_ICONEXCLAMATION);

break;

Linking with ASIOHost DLL

In the ASIOHost folder you will find two different ways of getting the exported functions from ASIOHost. You may use explicit or implicit dynamic linking with ASIOHost.dll at run time. The folders ExplicitLinking and ImplicitLinking each contain a little discussion about the form of linking under consideration, and the folder ExplicitLinking also contains source code for a loader class that employs LoadLibrary and GetProcAddress to get the functions from  ASIOHost.dll. Each of these folders has an identical copy of the header file ASIOHost.h describing the functions exported from ASIOHost.dll.

If you are developing your application in C++, the simplest thing to do is to use implicit linking with ASIOHost.lib set as an additional linker dependency in the linker properties of the compiler for your application. If you have to use explicit linking, class ASIOAudio, as defined in ASIOAudio.h and implemented in ASIOAudioInit.cpp is provided as an example of how to do this. Your application would typically provide a variable to be used as a pointer to a class ASIOAudio object, like this: LPASIOAUDIO pASIOAudio = NULL; At app init time you can create class ASIOAudio object using the function provided in ASIOAudioInit.cpp and passing it the address of your class ASIOAudio object pointer like this...

bLoadedASIOHost = LoadASIOHost(&pASIOAudio, hwndApp, "MyASIOAppName");

If that call was successful, you now have access to all functions exported by the DLL via pointer pASIOAudio. If you are linking ASIOHost into a .Net project, take note of the function declarations in ASIOAudio.h in the ExplicitLinking folder.

Demo Applications

ASIOHostDemo generates a sine wave test tone into outputs 0 and 1 of the selected device. It also demonstrates the different modes available, selectable from the menu. (see “Selecting Modes, above). If you are recording, captured audio data from inputs 0 and 1 of the selected device will be found in files “left.pcm” and “right.pcm” for mono mode or “stereo.pcm” for stereo mode in the application folder after your recording session. These are raw data files without a RIFF structure. Patching outputs 0 and 1 to inputs 0 and 1 of your ASIO sound card and choosing stereo play/record mode would be a good demonstration of the full duplex capabilities of ASIOHost.

Also included is an application called ASIOTest. This was used extensively during the development of ASIOHost. It will play mp3 or wave files and record at the same time if desired. It has a menu for selecting the mode and can thereby demonstrate operation in any mode. Furthermore it has a crude and ugly UI that allows you to Pause and Resume by clicking on those words displayed at the lower left and right corners of the screen, and you can set the volume by clicking in the central area in steps as indicated by the horizontal lines drawn on the screen. Clicking at the very bottom is full volume while clicking increasingly higher up results in increasingly lower volume. This application may not work with all sound cards, because I never put the effort into it to ensure that it would. Specifically, it doesn't use a buffer size that is compatible with the requirements because it is more concerned with creating a buffer size that is compatible with decoding mp3 frames. Like I said, I just used it for testing. I am sure you will make a prettier application than that.

SourceCode

The source code folder contains the full VS2008 project for ASIOHostDemo, and as well you will find just the source code alone in the Source folder. ASIOHostDemo.cpp is the main project file that illustrates how to employ the functions from ASIOHost as defined in ASIOHost.h, to implement an audio application. Also included in the MultiChannelDemo folder is a version of  ASIOHostDemo with Class VirtualChannel as an example of how one might go about accessing many channels with the full professional version of ASIOHost.

ASIOHost: The Free Version

Included in the SDK is a free version of the DLL both to permit evaluation without obligation, and for the pleasure of hobbyists who would like to develop audio applications for ASIO devices. The free version provides only a subset of the capabilities of the full licensed version, but its performance is not diminished in any way. It provides the same efficient audio path to and from the device that the full licensed version provides for 16 bit samples.

The free version is offered for your use on one condition – it may not be employed in a commercial application, or in exchange for money or other consideration. You need a licensed version for that.

The free version is restricted to 16 bit audio in 2 virtual channels in mono modes or 1 virtual channel in stereo mode, but any mode may be selected – thus offering full duplex operation.

The following functions are not supported in the free version. You may call them, but they do nothing, and return no useful information...

extern bool ASIOSetSampleRate(DWORD dwSampleRate);

extern DWORD ASIOGetOutputLatency(void);

extern DWORD ASIOGetInputLatency(void);

extern BITDEPTH ASIOGetBitdepth(void);

extern bool ASIOGetChannelName(char *name, int channel);

extern void SetMaxVirtualBuffers(int nMaxVirtualBuffers);

The free version of ASIOHost outputs a debug log with abundant information about the ASIO device – such as indicated by the above ASIO functions, and any error messages. It should be noted that you can set the sample rate in the control panel of your ASIO device.